# Additional Information — Smart Bottle

Date: 2024-01-12   |   Supplement to main Invention Disclosure

# A. Prototype Bench-Test Results (Prototype v0.3, December 2023)

| Metric | Achieved | Target |
| --- | --- | --- |
| Temperature accuracy | ±0.4 °C | ±0.5 °C |
| Volume accuracy | ±1.8 ml | ±2 ml |
| BLE range (open air) | 42 m | ≥ 30 m |
| Battery life | 16 days | ≥ 14 days |
| Water-resistance | IP67 pass | IP67 |

# B. Manufacturing Notes

The TPU gasket is injection-moulded to a ±0.05 mm tolerance to achieve IP67. Each load cell is epoxy-bonded and individually calibrated on the production line using a two-point calibration (0 g and 500 g reference weights). The Nordic nRF52840 module is FCC/IC pre-certified, reducing regulatory lead time.

# C. Regulatory Considerations

FCC Part 15 (BLE radio) — pre-certified module; declaration of conformity required.

FDA food-contact materials compliance for inner stainless-steel surface coating.

CE marking for EU distribution — EN 300 328 (BLE) + EN 62368-1 (safety); pending.

RoHS / WEEE compliance confirmed for all PCB components.

# D. Licensing Intent

The inventors intend to license core sensing + firmware IP to established drinkware brands. The UI/UX design of the mobile app may be separately licensed or kept proprietary. Defensive publication is acceptable for peripheral features (e.g., RGB LED hydration indicator).