# Patent Claims — Smart Hydration Monitoring Bottle

Application No.: [PENDING]   |   Filing Date: 2024-01-25

# Independent Claims

## Claim 1 — Apparatus

A portable hydration vessel comprising:
  a vessel body defining an interior volume configured to hold a liquid;
  a temperature sensing element configured to measure a temperature of the liquid;
  a volume sensing element configured to measure a quantity of liquid in the vessel body;
  a processor operatively coupled to both sensing elements; and
  a non-transitory memory storing instructions that, when executed by the processor, cause the processor to generate hydration data based on the measured temperature and the measured quantity of liquid.

## Claim 3 — Apparatus with Wireless Communication

The portable hydration vessel of Claim 1, further comprising:
  a wireless communication module configured to transmit the hydration data to an external computing device;
  wherein the wireless communication module operates according to a short-range wireless protocol.

## Claim 5 — Method

A computer-implemented method for hydration monitoring, comprising:
  receiving, from a sensor module of a portable vessel, temperature data and volume data;
  computing a hydration deficit based on the volume data and a user-defined daily hydration target; and
  generating a notification when the hydration deficit exceeds a predetermined threshold.

# Dependent Claims

## Claim 2

The portable hydration vessel of Claim 1, wherein the temperature sensing element comprises a negative temperature coefficient (NTC) thermistor.

## Claim 4

The portable hydration vessel of Claim 1, wherein the volume sensing element comprises a strain-gauge load cell configured to measure a weight of the liquid.

## Claim 6

The method of Claim 5, wherein the daily hydration target is computed as 35 millilitres per kilogram of a user's body weight.